# Commissioning and LMK generation for Thales PS 10K HSM at Ipoteka Bank

Document control

| Version | Content | Author | Date | approved |
| --- | --- | --- | --- | --- |
| 1.0 | Original version |  | . |  |

## Step 1 -- Environment check

| Action | Command | Result |
| --- | --- | --- |
| The hardware environment used for this task consists of a PC running Windows 7 operating system and terminal emulation. The Thales HSM is connected to this PC over serial line. A matrix printer without ribbon and loaded with PIN envelopes is connected directly to the HSM. The PC is not connected to any network and continuously is stored in the secure room under dual control. The security officer has verified that there is no tapped device in the PC. |  |  |

## Step 2 -- Device check

| Action | Command | Result |
| --- | --- | --- |
| The Thales HSM was stored in original packaging in secure room. The security officer has verified that the HSM is physically not harmed. The genuineness of the HSM was checked by running the "VR" command, the result of it was the following: | VR |  |

Example:

Online>vr

Base release: 3.1a

Revision:     1110-0831

Build Number: 0008

Bootstrap Version:       2.5.7

Kernel Version:          3.8.3

ESS FPGA Version:        2.44.3

HSM Core API Version:    3.4.14

HSM Application Version: 3.4.14

HSM COMMs Version:       1.9.2

Application Checksum:    FB90

Host Configuration: Async, Ethernet, SNA-SDLC 3274

Serial Number:  A3453266741E         Licence Issue No:  1

Performance:    20 TPS               Build location:    01

Base Software:  Version 3            Ship Counter:      1

Crypto:         3DES, RSA

Press "Enter" to view additional information...

HSM8-LIC001 V3 Base

HSM8-LIC002 RSA

HSM8-LIC008 Message Encryption

HSM8-LIC016 Basic EMV-based Data Preparation

## Step 3 – Authorization

| Action | Command | Result |
| --- | --- | --- |
| At first the HSM was switched to secure state using the two HSM physical keys. | -na- |  |

## Step 4 – Card formatting

| Action | Command | Result |
| --- | --- | --- |
| After this <N> smart cards were formatted. The key custodians, deputy key custodians and the authorization officers defined their own PIN. | FC |  |

Example:

Secure>fc

Insert card and press ENTER:

Format card for HSM SETTINGS/LMKs? [H/L]: l

Erasing card

Formatting card...

Enter new PIN for smartcard: ****

Re-enter new PIN: ****

Enter time [hhmmss]: 165600

Enter date [DDMMYY]: 251110

Enter User ID: LMKA1

Enter Issuer ID: DRB

Format complete

## Step 5 – LMK component generation

| Action | Command | Result |
| --- | --- | --- |
| After this 3 LMK components were generated, and they were stored on 3 smart cards. The key custodians took care of their own smart cards. | GK |  |

Example:

Secure>gk

Variant scheme or key block scheme? [V/K]: v

Key status? [L/T]: l

LMK component set [1-9]: 3

Enter secret value A:

Enter secret value B:

Enter value C:

Insert blank card and enter PIN: ****

Writing keys...

Checking keys...

Device write complete, check: 3333 94

## Step 6 – Authorization card creation

| Action | Command | Result |
| --- | --- | --- |
| As the next step two Authorizing Officer Smartcards were created using the CO command and two LMK component card. The Authorization Officers took care of their own smart cards. | CO |  |

Example:

Secure>co

Insert card for component set 1 or 2 and enter PIN: ****

Insert card for authorizing officer and enter PIN: ****

Copy complete

## Step 7 – Smart card formatting

| Action | Command | Result |
| --- | --- | --- |
| After this 5 smart cards were formatted. The deputy key custodians and the deputy authorization officers defined their own PIN. | FC |  |

Example:

Secure>fc

Insert card and press ENTER:

Format card for HSM SETTINGS/LMKs? [H/L]: l

Erasing card

Formatting card...

Enter new PIN for smartcard: ****

Re-enter new PIN: ****

Enter time [hhmmss]: 171000

Enter date [DDMMYY]: 251110

Enter User ID: LMKA2

Enter Issuer ID: DRB

Format complete

## Step 8 – LMK component duplication

| Action | Command | Result |
| --- | --- | --- |
| Using the original LMK components duplicates were created using the DC command. The deputy key custodians took care of their own smart cards. | DC |  |

Example:

Secure>dc

Insert card to be duplicated, enter PIN: ****

Insert blank card and enter PIN: ****

Writing keys...

Checking keys...

Device write complete, check: 3333 94

Make another copy? [Y/N]: n

1 copies made

## Step 9 – Dep. Authorization card creation

| Action | Command | Result |
| --- | --- | --- |
| As the next step two deputy Authorizing Officer Smartcards were created using the CO command and two LMK component card. The deputy Authorization Officers took care of their own smart cards. | CO |  |

Example:

Secure>co

Insert card for component set 1 or 2 and enter PIN: ****

Insert card for authorizing officer and enter PIN: ****

Copy complete

## Step 10 – LMK loading

| Action | Command | Result |
| --- | --- | --- |
| After this the newly generated LMK was loaded from the chip cards to the HSM using the LK command. | LK |  |

Example:

Secure>lk

Enter LMK id [0-4]: 0

Enter comments: Key comment

Load LMK from components

Insert card and enter PIN: ****

Check: 0441 62

Load more components? [Y/N]: y

Insert card and enter PIN: ****

Check: 7655 21

Load more components? [Y/N]: y

Insert card and enter PIN: ****

Check: 5687 41

Load more components? [Y/N]: n

LMK id: 00

LMK key scheme: Variant

LMK algorithm: 3DES(2key)

LMK status: Live

Comments: DRB Live

Confirm Details? [Y/N]: y

Use the LO command to load LMKs into key change storage.

## Closing records

All of the smart cards containing LMK components were placed into tamper evident envelopes.

| Component name | Name | CODE | Envelope serial number | signature |
| --- | --- | --- | --- | --- |

All of the Authorization Officer smart cards were placed into tamper evident envelopes.

| Component name | Name | CODE | Envelope serial number | signature |
| --- | --- | --- | --- | --- |

I confirm, that

- The device serial number is checked and matched with the serial number provided by the vendor before delivery

- The device is physically and logically checked, not found any harm or sign of tampering and it is in good condition.

| name |  | code | signature |
| --- | --- | --- | --- |
|  | Security Officer |  |  |
|  | Security Officer |  |  |

Budapest,